SEC. 16-6-X	CX-1 Corridor Mixed-Use District

- STATEMENT OF INTENT: This district is intended to promote a diversity of land uses to support walkable, mixed-use development patterns along business corridors and nodes served by municipal sewer and water.

- BASIC REGULATIONS

| Lot | Minimum Width | 150’ |
| --- | --- | --- |
|  | Minimum Area | 43,560 square feet |
|  | Minimum Open Space | 20%* |
|  | Maximum lot coverage | 80% |
| Principal Structure | Maximum Height | 60’ |
| Accessory Structures | Maximum Height | 30’ (or height of principal structure, whichever is lower) |
| Setbacks – All Structures | Street | 15’ |
|  | Rear | 15’ |
|  | Side | 15’ |

* Sec. 16-6-X(f)(1): A minimum of ten percent (10%) of the required open space shall be designed as usable open space.

- PERMITTED USES BY RIGHT (“PERMITTED USES BY RIGHT”)

- Multiple-family dwellings, more than three (3) units per structure.

- Licensed Community Living Arrangements.

- Retail establishments selling and storing product and merchandise.

- Professional Offices.

- Restaurants.

- Financial Institutions.

- Churches.

- Personal Service Establishments.

- State Licensed Massage Therapy.

- State Licensed Tattoo Piercing Studio.

- All Private Utility Installations provided all principal structures and uses are not less than fifty (50) feet from any residential district lot line.

- Governmental and cultural uses, such as fire and police stations, community centers, libraries, public emergency shelters, parks, playgrounds and museums.

- Public and parochial and private elementary and secondary schools.

- Rest homes, nursing homes, homes for the aged, clinics and children’s nurseries provided all principal structures and uses are not less than fifty (50) feet from any lot line.

- Licensed commercial day care centers.

- Pet grooming.

- Commercial Recreation Facilities.

- PERMITTED ACCESSORY USES (“ACCESSORY USES”)

- Renewable energy structures.

- Off-street parking for residential uses.

- Off-street parking and loading areas in conjunction with the operation of a business.

- Uses incidental to residential uses subject to Plan Commission approvals

- Uses incidental with the operation of a business subject to Plan Commission approvals

- OFF-STREET PARKING REQUIREMENTS

- Minimum Parking Space Requirements. Off-street vehicle parking space ("parking space") for this district shall be reduced, being provided for buildings and uses as hereinafter specified.

| Use Category | Minimum Spaces | Maximum Spaces |
| --- | --- | --- |
| Multi-family Dwelling Units | 1.2 per dwelling unit | No maximum |
| Apartments with Commercial Uses, Mixed Use Building | Per each individual land use minus 20% | Per each individual land use |
| Secondary and Elementary Schools | 1 for each 2 employees plus a reasonable number of stalls for student and other parking | No maximum |
| Rest Homes, Nursing Homes | 1 per 5 beds plus 1 per 3 employees | No maximum |
| Senior housing | 1 per dwelling unit | No maximum |
| Churches, Theatres, Auditoriums, Community Centers, and Other Places of Public Assembly | 1 per 5 seats | No maximum |
| Retail sales | 1 per 1,000 square feet of floor area | 5 per 1,000 square feet of floor area |
| Restaurants, Bars, Places of Entertainment, and Service Stores | Same as retail sales | No maximum |
| Professional Offices, Financial Institutions, Business, and Governmental | 1 per 500 feet for first 2,000 square feet of floor area, plus 1 per 1,000 square feet of floor area in excess of 2,000 square feet. | 4 per 1,000 square feet of floor area |

- Parking requirement reduced. A reduction of minimum parking space requirements may be requested by the applicant upon the submittal of a parking demand study. The study must include:

  - Operational characteristics by land use or building type such as but not limited to days and hours of operation, operational peak hours, expected maximum occupancy of residents, employees, or visitors and any other information necessary to determine the amount of parking stalls necessary for the building and use. Maximum occupancy for residential uses other than those listed in the parking supply schedule must include number of units, bedrooms per unit, and anticipated guest or management staff and resident population, limited to those who are expected to have vehicles needing parking.

  - Indication of the necessary parking supply by a unit of measure such as by square feet of floor area, number of occupants established by occupancy limits, or other applicable unit as recommended by the sources relied on to generate the proposal. Cite sources used to calculate parking supply needs such as the Urban Land Institute or Parking Generation Manual of the Institute of Transportation Engineers, latest edition.

  - Alternatively, a citation of comparable land use types or recent studies that are located within similar communities. Identify author of study, their credentials, location of precedents, and contact information for the community representatives where the parking was located. Applicants may use studies or information from their own operations in other jurisdictions if the information required here is provided.

- Surfacing. All off-street parking areas, loading areas, and access drives shall be surfaced with concrete, asphalt, permeable pavers, porous concrete, or other hard, dust-free surface approved by the Village Engineer and Public Services Director. Paved areas shall be maintained free of weeds, invasive species, dust, trash, and debris. Crushed stone and recycled asphalt are not acceptable surfacing materials.

- LANDSCAPING AND DESIGN

- Usable Open Space Required. A minimum of ten percent (10%) of the required open space shall be designed as usable open space, as defined in Section 16-18-1.

- Landscape areas. All unpaved yards of any lot in this district must be covered by either planting bed or landscaping.

- Planters and planting beds. All planters and planting beds must include shrubs, ornamental grasses (not turf or lawn grass), ground cover, annuals, or perennials.

- A minimum of four low shrubs, or eight perennials or ornamental grasses, are required per 100 square feet of landscaped area. Equivalent combinations of plants may be used, with two perennials or ornamental grasses equivalent to one low shrub. The remaining portion of the bed may be covered with natural, unpainted and undyed bark mulch, or natural, unpainted and uncolored, decorative stones, minimum 0.75 inch and maximum five inches in dimension.

- Annual beds must be maintained seasonally, replanting as necessary.

- Planting beds shall not include species considered noxious weeds per Section 8-1-2(a)(2).

- A recommended street tree list is maintained by the Planning & Zoning Department and is available upon request to guide species selection.

- Sidewalks required. Sidewalks shall be installed along all sides of the lot abutting a public street and shall be designed and built in accordance with the provisions of Title 6, Chapter 3.

- Interior parking lot landscaping. A minimum of one canopy tree and 100 square feet of landscape area is required for every ten (10) parking spaces or fraction thereof, provided that:

(a) Parking spaces within structures, designated motorcycle parking spaces and bicycle parking spaces are not included when determining the minimum amount of landscaping required.

(b) Trees and landscaped areas used to meet the screening requirements of this section are counted toward total parking lot landscaping requirements.

(c) Green infrastructure incentives. The interior parking lot landscaping requirement of this division may be reduced to not less than one tree and 100 square feet of landscaped area per thirteen (13) parking spaces if the following amounts of green infrastructure features are provided as a substitute for trees and landscaped areas:

1. Permeable paving. One hundred square feet of permeable paving may be substituted for one tree and 100 square feet of landscaped area.

2. Rain gardens. One hundred square feet of rain garden with a rainwater catchment area of at least 1,000 square feet may be substituted for one tree and 100 square feet of landscaped area.

3. Bioswales. Fifty cubic feet (375 gallons) of bioretention capacity with a rainwater catchment area of at least 1,000 square feet may be substituted for one tree and 100 square feet of landscaped area.

- Pedestrian walkways. The design of pedestrian walkways shall follow the Village Design Guidelines per Section 16-4-4.

- SCREENING

- Per Village Design Guidelines, the design of trash and recycling receptacles shall use similar grade materials as proposed on the principal structure, or a combination of landscaping, grading, and fencing. Receptacles shall be screened on three sides by an opaque masonry wall or fence with a maximum height of six (6) feet. Materials that are not solid, such chain-link fencing with slats, are prohibited.

  - Enclosure openings shall be gated with an opaque material and shall not be directly visible from a public right-of-way and/or adjoining residential areas.

- The design and screening of garage doors, loading areas, service entries and areas, and mechanical equipment shall follow the adopted Village Design Guidelines per Section 16-4-4.

- EXTERIOR LIGHTING

- The design of exterior lighting shall follow the provisions of Section 16-10-4 and the Village Design Guidelines per Section 16-4-4.

Definitions:

Planting bed. shall mean a designated area for the cultivation of plants such as shrubs, ornamental grasses, or similar vegetation, which is physically separated from lawn or turf areas. A planting bed may be located at grade or raised above the surrounding soil and is defined by a frame, stone borders, or similar.

Usable Open Space. A primarily contiguous outdoor or unenclosed area designed and accessible for outdoor living, recreation, pedestrian access, or landscaping. Such areas may include wetlands or ponds but shall exclude parking facilities and interior parking lot landscaping, driveways, utility or service areas, required street yards, and any space with a dimension of less than six (6) feet in any direction.